COEN 6541: Functional Hardware Verification

Verification Report for Calc1 & Calc2

Date of submission –April 24th, 2019

------------------------------------------------------------------

SUBMITTED BY

| Sreekanth Reddy Akkala | Anil Kumar Bandi | Sandeep Chanda |
| --- | --- | --- |
| (Student ID: 40070952) | (Student ID: 40036715) | (Student ID: 40064690) |

SUBMITTED TO

Prof. Dr. Otmane Ait Mohamed

# Contents

Table of figures	2

Introduction	3

Design Specification of Calc 1	3

Design specification of Calc 2	4

Testbench used for Verification	5

Transaction	5

Generator	6

Interface	6

Driver	6

Monitor	6

Scoreboard and Checker	6

Environment	6

Test	6

Test Scenarios	7

Test case 1	7

Test case 2	8

Test case 3	9

Test case 4	10

Test case 5	11

Test case 6	12

Test case 7	13

Test case 8	14

Test case 9	15

Test case 10	16

Test case 11	17

Current progress on Calc2	17

Bug Report	19

Conclusion	19

References	20

# Table of figures

Figure 1 Calc1 input/output description	4

Figure 2 Calc2 input/output description	4

Figure 3 Testbench for the design	5

Figure 4 Transcript for four concurrent addition command	7

Figure 5 Output Waveform for four concurrent addition command	7

Figure 6 Transcript to show that no port has higher priority	8

Figure 7 Output Waveform to show that no port has higher priority	8

Figure 8 Transcript for higher order 27 bits are ignored testcase	9

Figure 9 Output Waveform for higher order 27 bits are ignored testcase	9

Figure 10 Transcript for testcase of overflow condition	10

Figure 11 Output Waveform for testcase of overflow condition	10

Figure 12 Transcript for addition of two numbers whose sum is FFFFFFFF	11

Figure 13 Output Waveform for for addition of two numbers whose sum is FFFFFFFF	11

Figure 14 Transcript for subtraction of two equal numbers	12

Figure 15 Output Waveform for subtraction of two equal numbers	12

Figure 16 Transcript for testcase of underflow condition	13

Figure 17 Output Waveform for testcase of underflow condition	13

Figure 18 Transcript for shift by 0 places testcase	14

Figure 19 Output Waveform for shift by 0 places testcase	14

Figure 20 Transcript for shift by 31 places testcase	15

Figure 21 Output Waveform for shift by 31 places testcase	15

Figure 22 Transcript for illegal commands	16

Figure 23 Output Waveform for illegal commands	16

Figure 24 Output Waveform for the reset condition	17

Figure 25 Transcript for Calc2	18

Figure 26 Output Waveform for Calc2	19

Introduction

The main objective of this project is to write a verification functional test plan  based on a design  specification, creating  a verification  environment  for  the  given   design  using  the  test plan  so that  it  can be reusable  for multiple  designs  and to find  errors in  the design.  In this  project, Calc  1, Calc 2and Calc 3 designs are verified using different verification  tests  in  the  same environme nt that is created. Different test scenarios were tested and detailed description  of  the  results  are provided. System Verilog is used to code the project and is performed on Questasim.

Design Specification of Calc 1

Calc  1 design  specifications  include  handling  4 requests  having  equal  priority  in  parallel. It can perform 4 functions Add, Subtract, Shift left and Shift Right. Each port must wait for its response before sending in the next command. Works on FCFS algorithm and allows only 1 add/subtract  and 1 left/right  shift  operation  to perform  at a time.  Should  use  only  the  lower  order 5 bits of the second operand for shift operation and arithmetic operations are unsigned.

## Pin Description:

c_clk: Gives the clock to drive the input command, data/get the output data, response.

reqX_cmd_in<0:3>: It is the command  bit  that  defines  the  operation  to be performed  with  the  data provided where

‘0’ specifies No Operation

‘1’ specifies addition of Operand 1 and Operand 2

‘2’ specifies subtraction of Operand 1 and Operand  2 ‘5’ specifies Shift left Operand 1 by Operand  2 places ‘6’ specifies Shift right Operand 1 by Operand 2 places

reqX_data_in<0:31>: Operand 1 data arrives with a command bit and Operand 2 arrives on the following cycle.

reset<0:7>: Resets the data to ‘1111111’b at the start of test case for seven cycles. Outputs are ignored during this period.

out_respX<0:1>: 2-bit output response bits defines the operation performed at the output where

‘0’ specifies No response

‘1’ specifies successful operation completion

‘2’ specifies invalid command or overflow/underflow error ‘1’ specifies internal error

out_dataX<0:31>: Gives corresponding valid data accompanied by response.

Figure 1 Calc1 input/output description

Design specification of Calc 2

Calc 2 design  specifications  are similar  to that  of Calc 1 except  each port can handle  up to 4 operations at a time. So, a total of 16 commands are  handled  at a time.  Each  command  from  a port is sent one at a time, but the calc log may respond “out of order” depending  upon the interna l state of the queues. A 2-bit tag is accompanied with the command provided so as to track the corresponding data at the output.

Figure 2 Calc2 input/output description

## Pin Description:

reqX_tag_in(0:1): The tag bus is the two-bit identifier for each command from the port. The port requestor can reuse the tag as soon as the Calc2 HDL responds to the command.

out_tagX(0:1): The output tag bus corresponds to  the  command  tag  sent  by the  requester.  It is used to identify which command the response if for.

Testbench used for Verification

Transaction

Figure 3 Testbench for the design

The transaction  class that we created generates  the  randomized  stimulus.  The  stimulus  can be constrained to get the required values/range. Command and tag bits are also generated in this class respectively for all the ports. The expected tag bits, captured  tag  bits, output data bits, expected data bits, output response bits, expected response bits, are also declared in this class. Copying current random values for constraints not to repeat the values are also done in this class.

Generator

Generator class that we created randomizes the transaction class that is coded  to  create multiple input stimulus to be driven to DUT via driver class. This class adds a control to the loop,

  - ., the number of iterations done by the loop is controlled by declaring a variable. Also displays whether the randomization succeeded or not. Mailbox,  gen2drive  is  generated  in  this  class  to send the randomized stimulus from generator class to the driver. An event is also declared to note the completion of transactions.

Interface

Multiple signals in a device are placed in a single band called interface for easy handling, reusability of the input-output signals. In the  interface  class,  all  the  input-output  signals  are  declared. Clocking blocks for the monitor and the test class are also declared at the posedge of the clock in this class. Also, the direction of the signals for monitor, DUT and test are declared in the modport in this class.

Driver

The driver class creates a mailbox driv2scb in addition to the mailbox gen2driv. A custom constructor is created in order to pass the values  of the mailbox.  The  instance  of the virtual  interface is declared in this block. Timing information is  defined  within  the task provided  in  the class  for all  the 4 ports. Reset is declared in this class. Expected  data, expected  response  and expected  tag bits are calculated in this class. The randomized stimulus is driven to the DUT.

Monitor

The monitor class creates a new virtual interface intf2 and mailbox mon2scb. A custom constructor is used to pass the values of the interface and mailbox to the scoreboard. Output data, Output response and output tag bits are captured in this class  for  all the  4 ports from  the DUT.  All the tasks to capture the data, response and tags are parallel.

Scoreboard and Checker

Scoreboard gets the expected data, response and tag bits  from  the driv2scb  mailbox  using the get() function and stores in it. It also gets the captured data from the DUT  through  mon2scb mailbox and compares them. It displays the error message  if there is an overflow,  underflow,  equal and inequal output values.

Environment

Instances of all the classes are created and components  are  connected  in  this  class. Generator, Scoreboard and driver iterations are done in this  and displayed.  All  the components  are run in parallel in this class using a fork-join.

Test

Test class instantiates a copy of the environment class and uses it to control  all  the components of the design. Environment class  is  invoked  in  this  class.  Number  of times  the  loop has to be iterated is declared in this class.

Test Scenarios: Test case 1

Across all ports (e.g., four concurrent adds do not interfere with each other),  check  that each command can have any command follow it without  leaving  the state of the  design  dirty,  such that the following command is corrupted.

Figure 4 Transcript for four concurrent addition command

Figure 5 Output Waveform for four concurrent addition command

## Explanation & Observation:

Values for both the operands  are  randomly generated  and  added simultaneously.  When four concurrent additions  are given, we’re getting unexpected outputs for 3 of the ports. There is an error in addition datapath.

Test case 2

Check that there is fairness across all four ports such that no port  has higher  priority  than  the  others.

Figure 6 Transcript to show that no port has higher priority

Figure 7 Output Waveform to show that no port has higher priority

## Explanation & Observation:

Random data is generated and multiple commands are given simultaneously. It is observed  that there is no priority among the four ports – All ports have equal priority.

Test case 3

Check that the high-order 27 bits are ignored in the second operand of  both  shift commands

Figure 8 Transcript for higher order 27 bits are ignored testcase

Figure 9 Output Waveform for higher order 27 bits are ignored testcase

## Explanation & Observation:

Higher order 27 bits are not ignored  in  the  second  operand.  In port 2, the expected  data is 4, but the resultant data is 0. We found a bug in this case and is explained in the bug report.

Test case 4

Data dependent corner case: Add two numbers that overflow by 2(“FFFFFFFF”X+2)

Figure 10 Transcript for testcase of overflow condition

Figure 11 Output Waveform for testcase of overflow condition

## Explanation & Observation:

We observed that the two numbers are overflown by 2 – Overflow operation is working as expected but the response is 1 instead of 2.

Test case 5

Data dependent corner case: Add two numbers whose sum is “FFFFFFFF”X

Figure 12 Transcript for addition of two numbers whose sum is FFFFFFFF

Figure 13 Output Waveform for for addition of two numbers whose sum is FFFFFFFF

## Explanation & Observation:

We observed that the Added two numbers – (FFFFFFFD + 00000002) gives the expected result (FFFFFFFF)

Test case 6

Data dependent corner case: Subtract two equal numbers

Figure 14 Transcript for subtraction of two equal numbers

Figure 15 Output Waveform for subtraction of two equal numbers

## Explanation & Observation:

We have observed that the transcript isn’t producing the expected results but the output waveform is giving the required results.

Test case 7

Data dependent corner case: Subtract a number that underflows by 1

Figure 16 Transcript for testcase of underflow condition

Figure 17 Output Waveform for testcase of underflow condition

## Explanation & Observation:

We observed that the subtracted number underflows by 1(00000008- 00000009=FFFFFFFF) as expected but we have found the error in response bits which  is  explained in the bug report in detail.

Test case 8

Data dependent corner case: Shift 0 places

Figure 18 Transcript for shift by 0 places testcase

Figure 19 Output Waveform for shift by 0 places testcase

## Explanation & Observation:

fffffffd if shifted by 0 places, expected data should be same, but the resultant  data  is 00000000. The bug found is explained in the bug report in detail.

Test case 9

Data dependent corner case: Left Shift 31 places

Figure 20 Transcript for shift by 31 places testcase

Figure 21 Output Waveform for shift by 31 places testcase

## Explanation & Observation:

We have observed that the data is left shifted by 31 places as expected.

Test case 10

Check that the design correctly handles illegal commands.

Figure 22 Transcript for illegal commands

Figure 23 Output Waveform for illegal commands

## Explanation & Observation:

For illegal commands, obtained result is 0 as expected, but the response is 0 instead of 2.

Which is explained in the bug report.

Test case 11

Check that the reset function correctly resets the design.

Figure 24 Output Waveform for the reset condition

## Explanation & Observation:

We have observed that the reset function holds high for 7 cycles.

Current progress on Calc2

We have given tag bits along with  random input  stimulus.  The  captured  data and  captured tag bits are matching for the first iteration and it is not matching from the second iteration.

Figure 25 Transcript for Calc2

Figure 26 Output Waveform for Calc2

Bug Report

| BUG | TITLE | EXPLANATION |
| --- | --- | --- |
| 1 | Higher order 27 bits are not ignored | In the test case 3, the 2nd operand is 20(Decimal – 32, Binary -100000), which has ‘00000’ as the first 5 bits, meaning the output is supposed to shift 0 places. But, the code is considering 6th bit as well (100000) for decimal value 32. It is working fine until decimal 31. |
| 2 | Overflow operation | Test 4 show that the overflow operation produces the error in the response. The response is 1, where as the expected is 2. |
| 3 | Underflow operation | Test 7 shows that the underflow operation produces the error in the response. The response is 1 instead of 2. |
| 4 | Shift by 0 places for 32 | Test 8 demonstrates that the shift by 0 places command is producing the error because DUT is considering some random operands. Similar to Bug1 |
| 5 | Illegal commands | Test 10 shows that the given the illegal commands the output response as 1 which should be 2. |

Conclusion

The environment is created for the given specifications  of Calc1  and  Calc2.  Test bench and test plan are also specified as per the requirements. A detailed bug report is also provided for Calc1.Though we haven’t  ful y  completed Calc2,  we  are getting  the  correct  outputs  except  for the synchronization errors. This project has helped us in learning how an industry design can be verified in real-time.

References

    - Lecture Notes

    - https://www.verificationguide.com/p/home.html

    - https://www.chipverify.com/systemverilog/systemverilog-simple-testbench

    - Online Resources